ELEVATOR CONTROLLER VLSI PROJECT

Member 2 Complete Technical Guide & Presentation Companion (Aligned with Official Slide Deck)

MNNIT ALLAHABAD | ECE B2 | VLSI Lab Project

Team Members: Tsering Angmo (20244165), Ujjwal Pratap Singh (20244166), Vaibhav Raj (20244167)

Member 2 Ownership: Move, Door & Overload Subsystem (elevator.v lines 270–368) | Slides: 7, 10, 11, 12, 16, 17, 18

# 1. Architectural Placement in Presentation Deck (Slide 7)

As shown on Slide 7 (System Block Diagram), the elevator system is partitioned into three clearly bounded functional domains:

- Member 1: Request Capture (3 arrays, req_count, st) & SCAN Decision Engine (scan_direction, next_floor, goto).

- Member 2: Physical Movement, Door State FSM, Overload Sensor Safety, and Serviced Request Clearance (direction, door_state, over_weight_alert, complete).

- Member 3: Peripheral Actuation & Display: 4-Phase Stepper Motor Sequencer (stepper_motor.v) and 7-Segment Decoder (seven_seg_disp.v).

[NOTE / TECHNICAL INSIGHT] The Core Relationship: Member 1 decides WHERE the elevator needs to go next (algorithmic brain). Member 2 physically EXECUTES that command by steering the motor, stepping through floor levels, handling safety overrides, operating the door dwell timer, and clearing serviced requests from memory.

# 2. The Ping-Pong Handshake Protocol (goto and complete)

Member 2's entire module is governed by an asynchronous handshake with Member 1:

- Step 1 (Activation): 1. SCAN evaluates memory arrays, identifies destination floor i, writes next_floor = i, and pulses goto = 1 (rising edge).

- Step 2 (Acknowledgment): 2. Functionality block awakens via posedge goto in its sensitivity list. First action: immediately writes goto = 0 to re-arm the edge detector.

- Step 3 (Floor Transit): 3. Comparator Check: Compares next_floor vs current_floor. If next_floor > current_floor, sets direction = move_up (2'b01), door_state = 0, waits #300, and increments current_floor + 1.

- Step 4 (Closed-Loop Stepping): 4. Feedback Wakeup: Changing current_floor wakes SCAN via always @(current_floor). SCAN verifies next_floor and pulses goto = 1 again. The elevator advances one floor at a time.

- Step 5 (Handback): 5. Destination Arrival: When next_floor == current_floor, door dwell cycle executes, serviced request is cleared from memory, and complete = 1 is asserted to hand execution back to SCAN.

# 3. Verilog Technical Concepts Behind Member 2's Logic

Professors will grill you on how this code is evaluated by the Verilog event-driven simulator:

| Verilog Concept | Code Line in elevator.v | Engineering Role & Simulator Behavior |
| --- | --- | --- |
| Event Sensitivity List | always @(posedge goto or wire_open_door or wire_close_door or over_weight) | Defines mixed asynchronous triggers. posedge goto catches scheduler commands; level changes on wire_open_door, wire_close_door, and over_weight wake the block for safety/overrides. |
| Blocking Assignments (=) | Lines 272–368 | Assignments execute in immediate program order. This allows procedural resetting of goto=0 before time delays (#300) are entered. |
| Delay Control & Timescale | `timescale 10ms / 1ms; #300, #15, #20 | Simulation time units: 1 unit = 10 ms. Transit delay #300 = 3000 ms = 3.0 s. Door dwell #15 = 150 ms. Door extension #20 = 200 ms. |
| Iterative Loop Timer | for(d=0; d<20 & ...; d=d+1) | Replaces a multi-state hardware timer counter with an algorithmic delay loop. The loop dynamically samples wire_close_door and wire_open_door every 150 ms. |
| Integer Zero-Extension | cf = current_floor; | cf (32-bit signed integer) zero-extends 2-bit current_floor to safely index memory arrays upflr_call_arr[cf] and downflr_call_arr[cf]. |
| Directional Array Clearance | upflr_call_arr[cf]=0 vs downflr_call_arr[cf]=0 | Implements SCAN policy: only the call matching current sweep direction is cleared, leaving reverse calls pending. |

# 4. Slide-by-Slide Technical Walkthrough (Slides 10, 11, 12)

## Slide 10: Movement and Door Logic

Slide 10 details the four primary operational branches:

- 1. Comparator: Compare: Evaluates next_floor vs current_floor. If equal, serves floor; if greater, sets direction = move_up (2'b01); if less, direction = move_down (2'b10).

- 2. Transit Delay: Travel Time: #300 delay (3.0 seconds) represents transit time between adjacent floors before updating current_floor = current_floor +/- 1.

- 3. Dwell Timing: Door Dwell: Standard dwell runs 20 x #15 = 300 time units = 3.0 seconds, holding direction = stop (2'b11) and door_state = open (1'b1).

- 4. Dynamic Overrides: Door Button Overrides: open_door executes an extended 20 x #20 (4.0s) dwell loop. close_door immediately terminates both loops because wire_close_door == 0 fails.

## Slide 11: Clearing Requests and Overweight

Slide 11 explains request retirement and safety interlocks:

- Safety Interlock: Overweight Protection (Priority 1): Wrapped at top level if(~over_weight) ... else if(over_weight). When over_weight = 1, immediately sets direction = stop, door_state = open, and over_weight_alert = 1.

- Directional Filter: Directional Request Clearing: If scan_direction == 1, clears upflr_call_arr[cf] and flr_req_arr[cf]. If scan_direction == 0, clears downflr_call_arr[cf] and flr_req_arr[cf].

- req_count Decrement: Atomic Counter Decrement: Decrements req_count by 2 if both hall call and cabin request were satisfied at cf; decrements by 1 if only one was active.

- complete Handoff: Completion Signal: complete = 1 triggers posedge complete in Member 1's SCAN block to start the next search.

## Slide 12: Timing and Direction Codes

Summary table of real-time parameters matching Slide 12:

| Event | Verilog Delay / Loop Code | Real-World Duration | Hardware Output State |
| --- | --- | --- | --- |
| Move One Floor | #300 current_floor = current_floor +/- 1 | 3.0 seconds | direction = 01 (UP) or 10 (DOWN), door_state = 0 |
| Normal Door Dwell | 20 x #15 loop | 3.0 seconds | direction = 11 (STOP), door_state = 1 (OPEN) |
| Extra Time (open_door) | 20 x #20 loop | 4.0 seconds extra | direction = 11 (STOP), door_state = 1 (OPEN) |
| Close Early (close_door) | wire_close_door == 0 breaks loop | Instantaneous (<150ms) | door_state = 0 (CLOSED) immediately |

# 5. Waveform Analysis: Slides 16, 17, and 18

Member 2 must directly present the simulation waveforms shown on Slides 16, 17, and 18:

## Slide 16: Simulation Output 1 (Movement & Stepper Rotation)

- Marker 1: Transit Safety: While current_floor transitions (e.g. from 0 to 1, and 1 to 2), door_state remains strictly 0 (closed).

- Marker 2: Arrival Assertion: The instant current_floor matches next_floor, door_state transitions cleanly to 1 (open).

- Marker 3 & 4: Stepper Motor Coupling: During transit (direction = 01), q[3:0] pulses actively in sequence 0011 -> 1001 -> 1100 -> 0110. On arrival (direction = 11), q holds static holding torque.

## Slide 17: Simulation Output 2 (Door Button Overrides)

- Left Highlight: open_door Pulse: When open_door asserts, door_state is held high for the full 4.0-second extension (#20 x 20), visible as a prolonged door open plateau.

- Right Highlight: close_door Pulse: When close_door asserts, the dwell loop breaks immediately, and door_state drops to 0 without waiting for timeout.

## Slide 18: Simulation Output 3 (Overweight Safety Interlock)

- Alarm Assertion: Sensor Assertion: When over_weight rises to 1, over_weight_alert immediately goes to 1.

- Emergency Override: Door & Motor Override: door_state is forced open (1), and motor is held at stop (direction = 11).

- Sensor Clearance: Auto-Recovery: When over_weight falls back to 0, over_weight_alert resets to 0 and normal elevator scheduling resumes.

# 6. Hardware Synthesis Nuances & Viva Defense (Slide 20)

[HARDWARE / SYNTHESIS DISTINCTION] Slide 20 Connection: Slide 20 explicitly lists 'Replace # delays with clock counters' as Future Work. This is because # delays cannot synthesize on FPGA silicon (Spartan-6). In physical hardware, a digital clock divider counting clock pulses (e.g. 150 million cycles on a 50MHz clock) must be used.

## High-Probability Viva Questions for Member 2:

- Question 1: Q: Why did wire_close_door initialize to 1? A: It was declared reg wire_close_door = 1. In simulation, elevator_tb.v drives close_door = 0 at time 0, which updates wire_close_door to 0 via always @(close_door), resolving the issue before any doors open.

- Question 2: Q: What if overload occurs while the car is moving? A: In this behavioral code, the process sleeps inside #300, so it only responds after the delay. In synthesizable RTL, overload is checked synchronously on every posedge clk, cutting motor power within nanoseconds.

- Question 3: Q: Why decrement req_count by 2? A: If both an outside hall call and an inside cabin request exist for that floor, one physical stop satisfies both passengers. Decrementing by 2 maintains perfect counter synchronization.